Notes on structure of Standard Panel Cell, circa 2019.

Main Task Program Schedule: Periodic at 10.0 ms, Priority 10, Watchdog at 500 ms

Festo Equipment refer to: Commissioning of E2M_EthernetIP.pdf

1.	MD01_ManualWorkCell

MainRoutine

DM00_Mapping

EP00_MachineState

EP01_Initialize

EP02_Run

EP03_Terminate

EP04_ClearFaults

EP05_Manual

2.	MD02_Elevator

MainRoutine

DM00_Mapping

EP00_MachineState

EP01_Initialize

EP02_Run

EP03_Terminate

EP04_ClearFaults

EP05_Manual

MD01_ManualWorkCell - DM00_Mapping

StartRequest

In EP00_MachineState: Not Initialized, Not Stopped, Not Faulted.

Stopped: ModuleState.Stopped is only asserted in one place, EP03_Terminate.  It is asserted when Terminate.CMD is asserted.

Terminate.CMD is asserted in EP00_MachineState on rung 2 when ModuleState StopReq OR StopButton OR SafetyRelay1.

ManualWorkCell

ManualWorkCell is an instance of the UDT Module_Control in the Controller Tags group (global).

Manual

Manual is an instance of the UDT StateControl.

ModuleState

ModuleState is an instance of the UDT Module_Control.

ModuleState.Running active when cell is initialized, not stopped, and not faulted.

Most state transitions occur in EP00_MachineState.

It appears that the purpose of the SystemDateTime[5].0 is to allow the start button lamp to flash off for one second (each minute?) when there are no faults and the machine is not running.

State Control

Same instances occur in both ManualWorkCell and Elevator

Initialize.CMD: energize and latch start pb.

When Initialize.CMD is not set Initialize.SEQ[0] is set to 0.

When Initialize.CMD is set and Initialize.SEQ[0] = 0 Initialize.SEQ[0] is set to 20.

Initialize.SEQ[0]: is set to 0 on the first scan.

It is held at 0 whenever Initialize.CMD is not set.

The first scan that Initialize.CMD is trueand Initialize.SEQ[0] = 0 Initialize.SEQ[0] is set to 20.

When Initialize.SEQ[0] = 20 and the palm switch is closed Initialize.SEQ[0] is set to 30.

When Initialize.SEQ[0] = 30 ModuleState.Initialized is set.

The purpose of Initialize.SEQ[0] = 30 seems tp be to set ModuleState.Initialized = True.

ModuleState.Initialized latches itself until ModuleState.Stopped = True.